Job Hazard Analysis

| Task/Equipment: | Cleaning Suction Strainers |
| --- | --- |
| Department: | Powerhouse |
| Analyzed By: | Bill Partin & Bryana Borders |
| Date: | 4/12/19 |

Personal Protective Equipment Required:

Heavy Leather Gloves

Hard Hat

Trainings Required*/Recommended:

Proper use of impact tool*, OSHA 10

Equipment/Tools/Chemicals Required for the Job:

Socket wrenches, hand wrenches and rotary impact tool

| Tasks/Steps | Hazards Present | Safety Measures and Controls |
| --- | --- | --- |
| - Turn off pump at control panel | No hazards present | N/A |
| - Isolate strainers by closing suction and discharge stop valves | No hazards present | N/A |
| - Connect a hose to strainer blow down tap and run to floor drain | No hazards present | N/A |
| - Open drain valve and allow strainer cavity to drain and cool | Skin Burn | Use heavy leather gloves and allow strainer flange to cool |
| - Remove bolts from strainer flange and lower flange to floor | If you use an impact tool, this could cause wrist twist | Use heavy leather gloves for grip |
| - Remove wire mesh strainer | Finger cuts, sharp edges | Wear heavy leather gloves to prevent cuts |
| - Clean strainer using soap and water | No hazards present | N/A |
| - Replace strainer into cavity | Finger cut, sharp edges | Use heavy leather gloves |
| - While one person holds flange in place the other lines up and puts bolts in place | Finger pinch | Use heavy leather gloves |
| - Secure bolts using socket wrench or impact tool | Wrist twist | Be careful using wrenches and impact tool, use heavy leather gloves for grip |
| - Close drain valve | No hazards present | N/A |
| - Reopen isolation valves at suction and discharge stops | No hazards present | N/A |
| - Turn pump on at control panel and check for leaks | No hazards present | N/A |

Additional Information:

- Two Person Task

Lab Coat

Protection Suit

Fall Protection Harness

High Vis Vest

Safety Shoes

Splash Goggles

Cryogenic Gloves

Dust Mask

Respirator

Face Shield

Safety Glasses

Ear Protection